How To Develop the MD-328D using AVR Studio

- Precondition

- WinAVR installed

- AVR Studio installed

- Create a new project

Open the AVR Studio 4 from your program start menu.

Then select the "New Project".

Select the Project type: AVR GCC, and input project name. Then click Next.

Select the Debug platform: JTAGICE mkII and device: ATmega328. Then click "Finish" button.

Now you can see the new project opened as below:

- Add Include Files to the new project

Copy all the include files to the project directory, just as bolow:

Select "Add Existing Files", and add all the includes files to project.

Typing in the main codes.

You can compile the new empty project now.

- Add your codes in the project

Please remember to input the frequency in the project option. Here we using a 16MHz Ext crystal.

- Download the program to device

- How to install the driver of JTAGICE mkII

Install the WinDriver

Open the computer's device manager.

Select the "Operation" -> " Add outdated hardware"

Click "Next".

Select the second option.

Click "Next"

Click "Install from disk"

Click the "browse" button.

Find the AvrStudio4 directory of AVR Studio installed.

Select the file "windrvr6.inf" and click "Open".

Click "OK" button.

Select the "WinDriver" then click "Next".

Click "Next"

Make sure the check button selected, then click "Install".

Waiting the driver install complete.

Click "Finish", till now the windriver installed successfully.

You can check it in the device manager, as below:

Install the JTAGICE driver

Plug the JTAGICE mkII board to computer via USB port. Then the system will install the driver automatically. After it finished, you can see the JTAGIC mkII appear in device manager.

Then you can download your program from AVR Studio now just as we described above.